# Digital Twin Divergence Project Context For Another LLM

Current as of August 11, 2026. This file is intended as a compact handoff for

another LLM or collaborator. It explains what the project is, what has been

implemented, what is scientifically solid, what remains weak, and how the code

is organized.

# One-Sentence Summary

This project builds a security-aware digital twin for a Waveshare UGV01 tracked

rover, using encoder, IMU, GPS, timing, and AprilTag ground-truth data to study

when GPS should be trusted, down-weighted, or rejected under benign motion and

GPS-coordinate attack replay.

# Core Research Question

The project asks whether a rover digital twin can detect or bound divergence

when GPS measurements are unreliable or malicious, especially when adaptive

uncertainty policies can accidentally let attacked GPS influence the model's

own confidence.

The important distinction is:

- GPS is a sensor under test, not trusted ground truth.

- AprilTag/video tracking is the independent physical ground truth standard.

- Encoders and IMU provide protected relative-motion evidence.

- Edge timing and sequence counters provide packet-health evidence.

- The digital twin estimates rover state and decides whether GPS behavior is

consistent with the protected motion/timing evidence.

# Hardware State

Hardware used:

- Waveshare UGV01 tracked rover.

- ESP32-based onboard controller using the repo firmware path

`ugv01_gps_dev`.

- BN220 GPS module was integrated and worked earlier, but has recently been

disconnected due connector/wiring instability. The digital-twin and AprilTag

work can continue without live GPS until GPS attack experiments need fresh

GPS data.

- AprilTag visual tracking is now the preferred ground-truth route-fidelity

method.

Verified BN220 wiring when used:

BN220 white -> UGV01 RX

BN220 red   -> 5V

BN220 black -> GND

Do not change this wiring assumption casually. Earlier working behavior used

the UGV01 RX path.

# Firmware And Telemetry

Active firmware path:

ugv01_gps_dev

Important commands:

- `T:146`: GPS-only telemetry.

- `T:147`: combined UGV01 telemetry used by the edge logger.

- `T:404`: Wi-Fi station/AP configuration.

Example Wi-Fi update:

{"T":404,"ap_ssid":"UGV","ap_password":"12345678","sta_ssid":"YOUR_WIFI","sta_password":"YOUR_PASSWORD"}

The combined `T:147` packet includes:

- sequence counter and firmware timing: `seq`, `sample_ms`, `send_ms`,

`millis`

- motor/command state: `L`, `R`

- encoders: `enc_left`, `enc_right`

- voltage: `v`

- IMU: roll/pitch/yaw `r`, `p`, `y`; acceleration `ax`, `ay`, `az`; gyro

`gx`, `gy`, `gz`; magnetometer `mx`, `my`, `mz`; temperature `temp`

- GPS status: `gps_valid`, `gps_age_ms`, `gps_fix_type`, `lat`, `lon`,

`sat`, `hdop`, `alt_m`, `speed_mps`, `course_deg`

- GPS parser counters: `gps_chars`, `gps_sentences`, `gps_failed_checksums`

The edge logger adds:

- local wall and monotonic timestamps

- edge send, edge receive, edge midpoint, estimate, and alarm timestamps

- HTTP latency

- session-level clock-offset calibration

- stale-packet flag

- sequence-gap and request-failure counts

- queue depth

- run metadata such as route, surface, speed, attack type, trial, and square

calibration parameters

References:

- `docs/telemetry_protocol.md`

- `docs/log_data_dictionary.md`

- `docs/ugv01_esp32_bringup.md`

# Main Code Structure

Core digital-twin modules:

- `DigitalTwin/ekf.py`: EKF prediction and GPS update.

- `DigitalTwin/kinematics.py`: motion model support.

- `DigitalTwin/motion.py`: gyro-bias correction, encoder/IMU yaw diagnostics,

and slip indicators.

- `DigitalTwin/security.py`: GPS-independent predictor, covariance bounds, and

trusted/evidence-gated logic.

- `DigitalTwin/detector.py`: Mahalanobis/NIS scoring, detectability logic, and

confidence fields.

- `DigitalTwin/alarm.py`: robust initialization, motion gating, and persistent

alarm rules.

- `DigitalTwin/attack.py`: offline GPS attack injection.

- `DigitalTwin/analysis/real_data_study.py`: accepted-log study, benign

thresholding, attack campaign, bootstrapping, plots, and reports.

- `DigitalTwin/analysis/digital_twin_accuracy.py`: current sensor-agreement and

consistency report.

- `DigitalTwin/dashboard/`: visual replay dashboard for accepted logs.

Hardware/logging scripts:

- `bench_logger.py`: main `T:147` telemetry logger and scripted route support.

- `bench_logger_interactive.py`: terminal-driven manual motion commands.

- `bench_logger_square_0_5m.py`: 0.5 m square route runner.

- `bench_logger_square_1m.py`: 1 m square route runner.

- `bench_logger_curves.py`: figure-eight and S-curve timed route runner.

- `collect_rough_dataset.py`: helper for rough-surface benign data collection.

# Current Motion Script State

The UGV01 is a tracked rover, so turns are not perfectly repeatable. Recent

manual calibration showed that a slow encoder-count turn is better than

yaw-based or timed turning.

Current interactive turn settings:

90 deg turn target: 575 average opposite-track encoder counts

turn command:       0.038 to 0.045 is physically best; slower reduces overshoot

Current square settings in `bench_logger.py`:

SQUARE_STRAIGHT_FORWARD_CMD = (-0.10, -0.10)

SQUARE_TURN_CW_CMD          = (0.038, -0.038)

SQUARE_TURN_COUNTS_PER_90   = 575.0

SQUARE_SIDE_HOLD_SECONDS    = 1.5

SQUARE_CORNER_HOLD_SECONDS  = 4.0

SQUARE_STRAIGHT_WARMUP_CMD  = (-0.05, -0.05)

SQUARE_STRAIGHT_WARMUP_SECONDS = 0.6

Known issue:

- After a 90-degree turn, the rover sometimes pivots slightly before going

straight.

- Logs show transient one-track or uneven commands such as `L=0, R=-0.093` or

`L=-0.108, R=-0.151` before the clean straight segment.

- This is likely due to UGV01 motor ramping, HTTP command delay, track friction,

and residual settling after the turn.

- This is annoying for visually clean square execution, but it is not fatal for

the research because AprilTags measure the actual path.

For data collection inside a roughly `2 m x 1 m` area, prefer:

python bench_logger_square_0_5m.py --surface smooth --speed low --repeats 1 --trial N

python bench_logger_curves.py --route figure8 --repeats 2 --surface smooth_kitchen_floor --speed low --trial N

python bench_logger_curves.py --route s_curve --repeats 2 --surface smooth_kitchen_floor --speed low --trial N

If curves are too large:

--duration-scale 0.75

# Data Collected So Far

Bench and stationary data:

- `raw_logs/static/*.csv`: stationary GPS/static logs.

- `raw_logs/telemetry/*.csv`: combined `T:147` telemetry logs.

- Stationary GPS summary, telemetry latency/health reports, and replay review

pipelines exist under `DigitalTwin/analysis/`.

Benign route corpus:

- A 20-run accepted corpus was collected from `square0p5x3` runs.

- Conditions were two surfaces and two speeds:

smooth kitchen floor, rough permeable concrete, low speed, medium speed.

- These logs are useful for software replay, benign thresholding, and attack

replay, but they should be treated as a design/evaluation corpus rather than

final prospective validation.

AprilTag/video data:

- AprilTag ID 0 is mounted on the rover.

- Fixed world tags were placed around the route.

- ChArUco footage was collected for camera calibration.

- Important videos live under `docs/footage/`.

- Printable AprilTag and calibration files live under `docs/printables/`.

Existing high-confidence usable ground-truth motion data is limited. Earlier

estimates found only several minutes of useful synchronized AprilTag motion, so

more ground-truth footage is needed for final fidelity claims.

# AprilTag Ground Truth

AprilTags are needed because indoor GPS on a small route is not a reliable

physical accuracy reference. A 0.5 m square route can show around meter-level

EKF-to-GPS disagreement indoors, but that is sensor disagreement, not proof that

the rover's true path is wrong by a meter.

Ground truth role:

- AprilTag/world tags define a metric coordinate frame.

- The rover tag gives measured rover position and heading in video frames.

- The EKF output can be compared to this independent physical path.

- This allows reporting position error, heading error, trajectory-shape error,

and local relative-pose error.

Current known tag geometry:

- Rover tag ID 0.

- Rover tag size: 8 cm.

- Rover tag is mounted near the center top of the rover.

- Tag elevation above floor: about 8.112 cm.

- Earlier reference layouts included a 1.5 m square and a measured trapezoid.

Useful accuracy metrics:

- ATE RMSE: global trajectory position error.

- Median ATE: typical global position error.

- P95 ATE: tail position error.

- RPE RMSE: short-window relative motion error.

- Heading MAE: heading-angle error.

- Path-length ratio: whether total traveled distance is close to truth.

- Percent within thresholds: samples within 0.10, 0.25, 0.50 m of ground truth.

Important caveat:

- Same-run calibration results are provisional.

- Final claims require training/calibration on some runs and evaluation on

untouched held-out video/log runs.

# Current Digital Twin Model

The digital twin is primarily an EKF-based tracked-rover model.

State:

x, y, heading

Tracked-drive update:

dL = left_meters_per_tick  * delta_left_ticks

dR = right_meters_per_tick * delta_right_ticks

ds = (dL + dR) / 2

dtheta = (dR - dL) / effective_track_width_m

The project has moved away from "wheel radius/wheelbase" wording because UGV01

is tracked, not a normal wheeled differential-drive robot. Use:

left_meters_per_tick

right_meters_per_tick

effective_track_width_m

heading_sign

Current tracked-turn findings:

- Vendor/nominal physical track width is not sufficient for turn prediction.

- AprilTag calibration suggests effective tracked-turn width around `0.192 m`.

- Earlier encoder-only estimate was around `0.1818 m`.

- The difference is expected because tracked vehicles slip laterally during

turns.

IMU status:

- IMU yaw/gyro is useful as an independent evidence and slip-diagnostic signal.

- Direct high-weight gyro fusion degraded some trajectory fidelity in existing

logs.

- Current direction is conservative: estimate gyro bias, use IMU for

diagnostics/evidence, and only fuse carefully if validated against AprilTag

ground truth.

# Accuracy Status

Do not overclaim digital-twin physical accuracy from GPS-only comparisons.

Existing EKF-to-GPS metrics quantify sensor agreement, not true physical

accuracy. They are useful for consistency checks but unreliable as final

fidelity evidence indoors.

Earlier sensor-agreement values included roughly:

EKF-to-GPS RMSE:             about 1.08 m

Median EKF-to-GPS difference: about 0.67 m

Samples within 1 m of GPS:    about 70.6%

Samples within 2 m of GPS:    about 94.0%

These are not good enough as physical accuracy claims for a small indoor route

because indoor BN220 GPS can drift and wander. The right answer to an advisor

is:

> The existing indoor GPS logs are useful as degraded-GPS stress tests, not as

> digital-twin fidelity ground truth. Fidelity will be measured against

> AprilTag ground truth using position, heading, and trajectory-shape errors.

Current AprilTag-calibrated results are promising but provisional because they

partly use same-run calibration. They indicate the model can improve

substantially when tracked-turn geometry is calibrated, but final validation

needs held-out footage.

# Attack And Threat Model

The current attack campaign is offline replay injection, not live RF GPS

spoofing.

Main attacker assumption:

- The attacker can alter GPS coordinates delivered to the edge analysis stream.

- The attacker does not control encoders, IMU, sequence counters, or edge

timing evidence in the core threat model.

- Raw physical logs and rover behavior remain unchanged; attacks are injected

during replay.

Attack families:

- Step bias: sudden GPS position jump.

- Slow drift: GPS gradually drifts in along-track or cross-track direction.

- Freeze: GPS coordinate becomes stuck.

- Replay: earlier GPS segment is replayed.

- Strategic/offline attacker variants may choose timing or direction based on

replay knowledge.

Important terminology:

- Do not call all replay rows "independent physical attacks."

- Use "attack-run-start combinations" and "detector-run evaluations" carefully.

- "Harmful-but-stealthy" should be phrased more cautiously as

"tolerance-exceeding paired divergence before alarm" unless physical ground

truth is present.

# Baselines

Baseline detector/model families used or discussed:

- GPS jump detector: detects sudden unrealistic jumps between GPS positions.

- Fixed NIS / chi-square Kalman innovation test: classic innovation

consistency check.

- Innovation-gated EKF: rejects or down-weights GPS when innovation is too

large.

- Huber EKF: robust update that reduces influence of large residuals.

- CUSUM: accumulates persistent smaller residuals.

- Naive adaptive covariance: changes uncertainty based on residuals, but can be

vulnerable to GPS-residual feedback.

- GPS-independent or evidence-gated adaptation: limits adaptation using

non-GPS evidence such as IMU/timing/sequence behavior.

The project's core argument is not "EKF beats all baselines." The stronger

argument is:

> Adaptive uncertainty can create a feedback path from attacked GPS to model

> confidence. A security-aware digital twin should separate GPS evidence from

> protected motion/timing evidence and explicitly bound when GPS can influence

> uncertainty.

# Novelty Position

The project is not novel just because it uses an EKF, GPS, or a tracked rover.

Those are established.

More defensible novelty:

- It frames adaptive uncertainty itself as a possible security vulnerability.

- It separates operational GPS fusion from a GPS-independent security branch.

- It combines rover telemetry, packet timing, and physical motion evidence in a

digital-twin divergence detector.

- It uses a real low-cost tracked UGV with firmware-level timestamped telemetry,

not only simulation.

- It moves toward independent AprilTag ground truth to quantify digital-twin

fidelity before making attack claims.

- A future hybrid neural EKF could learn slip/turn corrections while preserving

interpretable covariance and security constraints.

Weak novelty if left unfinished:

- GPS-only attack replay without independent ground truth is not enough for a

top venue.

- Indoor small-route GPS disagreement cannot be presented as physical accuracy.

- A generic NN replacing EKF would be less defensible without much more data.

# Recommended Model Direction With Neural Networks

Do not replace the EKF with a full black-box neural net. The better path is a

hybrid neural EKF:

- Keep EKF physics and explicit covariance.

- Use a small MLP or GRU to learn tracked-rover slip/turn corrections from

protected inputs.

- Inputs should be encoder deltas, `gz`, yaw diagnostics, commands, `dt`, and

surface label.

- Outputs should be bounded corrections to distance/heading and/or process

covariance scale.

- GPS should not be the training ground truth indoors.

- AprilTag trajectory should be the training/evaluation target.

Example target:

true_delta_distance - encoder_delta_distance

true_delta_heading  - encoder_delta_heading

Security constraint:

GPS may reduce its own trust, but should not be allowed to increase protected

model confidence without independent IMU/timing/encoder evidence.

This yields a cleaner research contribution:

> An evidence-gated, slip-aware neural EKF for secure low-cost tracked-rover

> digital twins.

# What Is Done

Implemented:

- Firmware path with `T:146`, `T:147`, and GPS/timing additions.

- Edge logger with timestamped telemetry, packet health, and session clock

calibration.

- Bench telemetry and stationary GPS analysis scripts.

- Hardware replay path through the digital twin.

- EKF, detector, alarm, uncertainty-policy variants, and attack replay.

- Real-data study pipeline with manifesting, thresholding, attack campaign,

bootstrapping, and plots.

- Dashboard for visual digital-twin replay.

- AprilTag printables, camera calibration work, overlay rendering, and initial

ground-truth trajectory processing.

- Tracked-turn calibration showing nominal geometry over-rotates; effective

width is larger.

- Motion scripts for interactive commands, square routes, figure-eight, and

S-curve routes.

- Several advisor-facing docs, manuscripts, and roadmap files.

# What Still Needs To Be Done

Highest priority scientific work:

1. Collect cleaner synchronized AprilTag ground-truth runs.

2. Split data into train/development, validation, and untouched final test.

3. Quantify physical digital-twin fidelity against AprilTag ground truth.

4. Recalibrate tracked-turn/slip model only on train/development data.

5. Evaluate on held-out AprilTag runs without retuning.

6. Regenerate digital-twin accuracy, threshold, and attack reports after the

final model is frozen.

7. Only then make strong claims about GPS attacks and detection performance.

High-value data collection:

- indoor smooth kitchen floor

- indoor carpet or rough indoor surface

- routes that stay within camera view: small squares, figure-eight, S-curve,

random/freeform path

- low and medium speed

- clockwise and counter-clockwise turns

- straight, reverse, stop-start, arcs, and mixed trajectories

Approximate data need:

- prototype: current data plus a few more clean videos is enough

- strong paper: roughly 40-60 minutes of synchronized usable motion minimum

- better: 60-90 minutes with held-out validation/test, balanced across surfaces

# Commands To Know

Run interactive manual motion:

python bench_logger_interactive.py --ip <UGV_IP>

Run a 0.5 m square:

python bench_logger_square_0_5m.py --surface smooth --speed low --repeats 1 --trial N

Run a figure-eight:

python bench_logger_curves.py --route figure8 --repeats 2 --surface smooth_kitchen_floor --speed low --trial N

Run an S-curve:

python bench_logger_curves.py --route s_curve --repeats 2 --surface smooth_kitchen_floor --speed low --trial N

Open dashboard:

python -m DigitalTwin.dashboard.server --open

Generate accepted-log study:

python -m DigitalTwin.analysis.real_data_study

Summarize an existing campaign:

python -m DigitalTwin.analysis.real_data_study --summarize-existing

Generate digital-twin accuracy report:

python -m DigitalTwin.analysis.digital_twin_accuracy

Run tests:

python -m pytest -q

# Publication Potential

Neutral assessment:

- As-is, without independent ground truth and prospective validation, this is

more like a strong project/prototype or workshop-level paper.

- With AprilTag ground truth, held-out fidelity evaluation, clean methodology,

and cautious attack replay claims, it can become a credible conference/journal

submission.

- For a top/very strong venue, the paper needs a sharper contribution:

security-aware uncertainty adaptation, evidence-gated GPS trust, and

physically validated tracked-rover digital-twin fidelity.

The strongest paper framing is not:

> We built a rover EKF and detected GPS spoofing.

It is:

> We show that adaptive uncertainty in a digital twin can create a

> measurement-to-confidence feedback vulnerability, and we evaluate an

> evidence-gated digital-twin architecture on a real tracked rover with

> independent visual ground truth.

# Guidance For Future LLMs

When helping with this project:

- Do not overclaim GPS as ground truth indoors.

- Do not call same-run AprilTag calibration a held-out result.

- Prefer tracked-drive terminology over wheel-radius/wheelbase terminology.

- Keep firmware behavior stable unless explicitly asked to modify it.

- Preserve BN220 wiring notes.

- Be careful with raw logs; many are development/debug runs.

- Generated analysis artifacts under `DigitalTwin/datasets/analysis/` may be

stale if model or policy code changed.

- If changing detector, model, or uncertainty policy, regenerate downstream

reports before quoting numbers.

- If changing route scripts, first inspect newest logs and only make small,

physically motivated changes.

# Current Bottom Line

The project has a substantial implementation: rover telemetry, edge logging,

EKF replay, attack injection, detector variants, dashboard, AprilTag tracking,

and route scripts all exist. The main remaining gap is not code volume; it is

scientific validation. The next major milestone is clean AprilTag-grounded

fidelity evaluation, followed by frozen-model attack replay and carefully

bounded claims.